SNMP exercises, part 1

intERLab / AIT March 2008

1. Getting packages

# apt-get install snmp

# apt-get install snmpd

# apt-get install mbrowse

2. GET and WALK

To control that your SNMP installation works:

- The backbone router and net routers

# snmpstatus -c 1nterl4b -v1 10.10.10.10

# snmpstatus -c 1nterl4b -v1 10.10.11.1

# snmpstatus -c 1nterl4b -v1 10.10.11.2

- The network switches:

# snmpstatus -c 1nterl4b -v1 10.10.1.253

# snmpstatus -c 1nterl4b -v1 10.10.2.253

- Try to snmpwalk different parts of these equipments' MIBs:

# snmpwalk -c 1nterl4b -v1 10.10.?.? 1.3.6.1.4.1.9.9.13.1.3 | more

# ...

a) Do all the devices answer ?

b) Do you notice anything important about the OID on the output ?

3. Configuration of snmpd

- Edit the following file:

# vi /etc/snmp/snmpd.conf

Comment the line (ADD '#' in front):

com2sec paranoid  default         public

And UNcomment the line (REMOVE the '#' in front):

#com2sec readonly  default         public

Edit the file /etc/default/snmpd, and find the line:

SNMPDOPTS='-Lsd -Lf /dev/null -u snmp -I -smux -p /var/run/snmpd.pid 127.0.0.1'

Remove 127.0.0.1 at the end, so you have:

SNMPDOPTS='-Lsd -Lf /dev/null -u snmp -I -smux -p /var/run/snmpd.pid'

- Restart snmpd

# /etc/init.d/snmpd stop

# /etc/init.d/snmpd start

4. Check that snmpd is working:

# snmpstatus -c public -v1 localhost

- What do you observe ?

5. Check now that you can run snmpstatus against your neighbor's server:

- Find out what your neighbor's IP is, ask them to run:

# ifconfig eth0

(your IP is 10.10.X.Y where X is 1 or 2, and Y is between 1 and 10.

- Check snmp against their machine:

# snmpstatus -c public -v1 10.10.X.Y

6. Adding MIBs

Remember when you ran:

# snmpwalk -c 1nterl4b -v1 10.10.11.1  1.3.6.1.4.1.9.9.13.1.3 | more

If you noticed, the SNMP client (snmpwalk) couldn't interpret

all the OIDs coming back from the Agent:

SNMPv2-SMI::enterprises.9.9.13.1.3.1.3.1 = Gauge32: 39

SNMPv2-SMI::enterprises.9.9.13.1.3.1.3.2 = Gauge32: 50

SNMPv2-SMI::enterprises.9.9.13.1.3.1.3.3 = Gauge32: 34

What is '9.9.13.1.3.1.3' ?

To be able to interpret this information, we need to download extra

mibs...

- Download the following files to your machine (NOTE: FTP access to Cisco.com doesn't work well from AIT, the normal URL is:

ftp://ftp.cisco.com/pub/mibs/v2/CISCO-SMI.my

ftp://ftp.cisco.com/pub/mibs/v2/CISCO-ENVMON-MIB.my

... but this time we download them from the local webserver)

# cd /usr/share/snmp/mibs

# wget http://noc/mibs/CISCO-SMI.my

# wget http://noc/mibs/CISCO-ENVMON-MIB.my

- Create the file /usr/share/snmp/snmp.conf, and put into it:

mibdirs /usr/share/snmp/mibs

mibs ALL

This tells the snmp* commands that they should load ALL mibs in the
    mibdir /usr/share/snmp/mibs

Save the file, quit.

Now, try again:

# snmpwalk -c 1nterl4b -v1 10.10.11.1  1.3.6.1.4.1.9.9.13.1.3 | more

What do you notice ?