IP/Unix Preparation Course

July 19, 2009

Exercises: Terminals and Consoles

- Open your first “virtual terminal” window:

Press the following key combination:

 	Alt-Ctrl-F1

What do you see?

Switch between virtual terminals 1 through 6. Once you are in a console window you can just press:

 	Alt-F[1-6]

to move between consoles.

Log in to a console as the user sysadmin:

- login: sysadmin
	Password: <given in class>

Take a look at what processes are running on your machine:

	$ ps auxwww

If you had to you could kill a process that might have hung your X session (Gnome). 

To return to Gnome (your GUI) press:

	Alt-Ctrl-F7

- Configure Minicom to work as a console for a router or switch

We want Minicom to be configured to use serial port ttyS0 at 9600 bps with 8N1 and 
no hardware flow control for use later in the week. As sysadmin open a terminal window:

	1.) $ sudo minicom
	2.) Ctrl-a,z, then Choose “cOnfigure Minicom”
	3.) Choose “Serial port setup”
	4.) Set Serial Device to be /dev/ttyS0
	5.) Set Hardware Flow Control to “No”
	6.) Exit, then press Ctrl-a, z. Choose “comm Parameters”
	7.) Set Speed to 9600
	8.) Exit. Exit Minicom (Ctrl-a, x) and restart Minicom:
	9.) $ sudo minicom