Consider the following simple network (very similar to example in talk, just with longer and more fiber links):

Use the tables below to develop pricing for a network based on gigabit Ethernet and 10gig Ethernet links for all fiber types (OM1, OM2, OM3, OM4, and G.652.D).

| Types of Optical Interfaces |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Standard | Speed |  | Supported Fiber |  |  |  | Cost |  |
| 100baseFX | 100Mbs |  | MM |  |  |  | $250 |  |
| 1000baseSX | 1000Mbs |  | MM |  |  |  | $500 |  |
| 1000baseLX/LH | 1000Mbs |  | MM or SM |  |  |  | $1,000 |  |
| 10GbaseSR | 10Gbs |  | MM |  |  |  | $1,500 |  |
| 10GbaseLRM | 10Gbs |  | MM |  |  |  | $1,300 |  |
| 10GbaseLR | 10Gbs |  | SM |  |  |  | $4,000 |  |
| 10GbaseER | 10Gbs |  | SM |  |  |  | $10,000 |  |
| Distances Supported by Interfaces |  |  |  |  |  |  |  |  |  |  |
| Standard |  | OM1 |  | OM2 | OM3 |  |  | OM4 |  | G.652.D |
| 100baseFX |  | 2km |  | 2km | 2km |  |  | 2km |  | No |
| 1000baseSX |  | 275m |  | 550m | 1km |  |  | 1.1km |  | No |
| 1000baseLX/LH |  | 500m |  | 500m | 500m |  |  | 500m |  | 10km |
| 10GbaseSR |  | 33m |  | 82m | 300m |  |  | 550m |  | No |
| 10GbaseLRM |  | 220m |  | 220m | 220m |  |  | 220m |  | No |
| 10GbaseLR |  | No |  | No | No |  |  | No |  | 10km |
| 10GbaseER |  | No |  | No | No |  |  | No |  | 40km |

| Cost of Fiber |  |
| --- | --- |
| Fiber Type | Budget Cost per km |
| OM1 | $4,884 |
| OM2 | $4,054 |
| OM3 | $10,151 |
| OM4 | $19,959 |
| G.652.D | $1,185 |

Here are some worksheets to use.

What is the total installed fiber length?

| Cost of 1 Gigabit Network |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fiber Type | Fiber Cost | Optics Used | Optics Cost | Total Cost |
| OM1 |  |  |  |  |
| OM2 |  |  |  |  |
| OM3 |  |  |  |  |
| OM4 |  |  |  |  |
| G.652.D |  |  |  |  |

| Cost of 10 Gigabit Network |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fiber Type | Fiber Cost | Optics Used | Optics Cost | Total Cost |
| OM1 |  |  |  |  |
| OM2 |  |  |  |  |
| OM3 |  |  |  |  |
| OM4 |  |  |  |  |
| G.652.D |  |  |  |  |